Fission Matrix Acceleration in Monte Carlo Simulations and Its Impact on Neutron Clustering

Andy Zhang, aqz22

Standard methods of Monte Carlo neutron transport are susceptible to non-physical intergenerational spatial correlations. Such ‘neutron clusters’ lead to slower convergences and worse statistics. Prior attempts to address this issue have led to marginal improvements whose effectiveness is limited to idealised systems. The following presentation proposes a novel method for clustering mitigation using ‘fission matrix acceleration’. This technique uses the preliminary tallies of region-to-region neutron production to estimate the analytic fission source. Fission matrix acceleration was previously employed to accelerate the convergence of the fission source, but its effect on neutron clustering is unknown. Numerical experiments were performed on two different systems: a homogeneous system, and a 3-region fissile-nonfissile-fissile system. Both visual inspection and statistical analysis of the resulting neutron source show a clear reduction in neutron clustering in several cases. However, clustering is worsened in situations with especially bad fission matrix statistics, such as weakly-coupled systems and low cycle numbers. Future work will be focused on mathematically deriving the experimental observations, and theoretically determining the optimal parameters of the fission matrix.